RESTRICTED

RHS 68 galley-countertop - Install procedure

DMC: BD700-A-J25-31-41-01AAC-720A-A

Language: sx/US

Issue no: 004

Issue Date: 2025-05-26

Title: RHS 68 galley-countertop - Install procedure

Security classification: Class02

Responsible partner Bombardier/3AB48

company:

| Originator: | Bombardier/3AB48 | G25028029 REV C, G25043792 REV NC, |
| --- | --- | --- |
| Applicability: | Model: BD700 |  |
| Technical | Authority information: |  |
| standard: |  | G25043794 REV NC |

Authority exceptions:

Authority notes: 	G25023214 REV J

Quality assurance first verificationsecond verification Cleared Table Top

status:

| Reason for updating: | This DM is updated to reflect engineering available at the time of authoring. |
| --- | --- |

See applicability on the 
first page of the DM

BD700-A-J25-31-41-01AAC-720A-A

BD700-A-J25-31-41-01AAC-720A-A 
	RESTRICTED 2025-05-26  Page 1

RESTRICTED

Intentionally left blank

See applicability on the 
first page of the DM

BD700-A-J25-31-41-01AAC-720A-A

BD700-A-J25-31-41-01AAC-720A-A 
	RESTRICTED 2025-05-26  Page 2

RESTRICTED

RHS 68 galley-countertop - Install procedure

Applicability: Model: BD700

Table of contents 	Page

RHS 68 galley-countertop - Install procedure..................................................................... 	1

References................................................................................................................................ 	1

Preliminary requirements......................................................................................................... 	2

Procedure.................................................................................................................................. 	3

Requirements after job completion........................................................................................ 	6

List of tables 	Page

1 References.................................................................................................................. 	1

2 Required conditions................................................................................................... 	2

3 Support equipment.................................................................................................... 	2

4 Consumables, materials, and expendables............................................................ 	2

5 Spares......................................................................................................................... 	2

6 Required conditions................................................................................................... 	6

List of figures 	Page

1 RHS 68 galley-countertop........................................................................................ 	4

References

Table 1 References

| Data Module/Technical Publication | Title |
| --- | --- |

BD700-A-J25-31-41-01AAC-520A-A BD700-A-J20-50-00-00AAA-711A-A BD700-A-J33-51-03-01AAA-720A-A

BD700-A-J51-24-00-00AAA-690A-A

BD700-A-J51-24-00-03AAA-621A-A

RHS 68 galley-countertop - Remove procedure Threaded fasteners - Tighten procedure 
Entrance-area emergency-lighting exit-sign - Install procedure 
Preparation of surfaces for adhesive-bonding and painting - Miscellaneous 
Bonding with Silicone adhesive - Bond

See applicability on the first page of the DM

BD700-A-J25-31-41-01AAC-720A-A

BD700-A-J25-31-41-01AAC-720A-A 
RESTRICTED 2025-05-26  Page 1

RESTRICTED

Preliminary requirements

Required conditions

Table 2  Required conditions

| Action/Condition | Data Module/Technical publication |
| --- | --- |
| Make sure the aircraft is in the same | BD700-A-J25-31-41-01AAC-520A-A |

configuration as the remove data module.

Support equipment

Table 3  Support equipment

| Name | Identification/Reference | Quantity | Remark |
| --- | --- | --- | --- |
| Torque wrench | std-0020 | 1 |  |

Consumables, materials, and expendables

Table 4  Consumables, materials, and expendables

| Name | Identification/Reference | Quantity | Remark |
| --- | --- | --- | --- |
| Silicone adhesive (RTV | 06-065 | As |  |
| 108 TY 4) | 07-001 | necessary |  |
| Isopropyl alcohol |  | As |  |

necessary

| Lint-free cloth | 05-001 | As |
| --- | --- | --- |

necessary

Spares

Table 5  Spares

| Name | Identification/Reference | Quantity | Remark |
| --- | --- | --- | --- |
| galley countertop | CSN 25-31-41- | 1 |  |

01AAE001

Safety conditions

None

See applicability on the first page of the DM

BD700-A-J25-31-41-01AAC-720A-A

BD700-A-J25-31-41-01AAC-720A-A 
	RESTRICTED 	2025-05-26  Page 2

RESTRICTED

Procedure

1 	For an illustration with this procedure, refer to Fig. 1.

2 	To install the galley countertop (CSN 25-31-41-01AAE001) (3), do the steps that

follow:

2.1 	Clean the faying surface with lint-free cloth (05-001) made moist with

isopropyl alcohol (07-001). Refer to BD700-A-J51-24-00-00AAA-690A-A.

2.2 	Put the galley countertop (CSN 25-31-41-01AAE001) (3) in position.

2.3 	Bond the internal edges of the galley

countertop (CSN 25-31-41-01AAE001) (3) on the galley with silicone

adhesive (06-065). Refer to BD700-A-J51-24-00-03AAA-621A-A.

Note

The silicone adhesive should flush with the galley

countertop (CSN 25-31-41-01AAE001) (3) for the horizontal gaps and

with the panels for the vertical gaps.

2.4 	Install the washers (5) and screws (4).

2.5 	With a torque wrench (Commercially available, Pt. No. std-0020),

tighten the screws (4) from 13 to 16 lbf·in (1,47 to 1,80 Nm). Refer to

BD700-A-J20-50-00-00AAA-711A-A.

2.6 	Install the sink cover (2).

3 	To install the cap assembly (11), do the steps that follow:

3.1 	Put the cap assembly (11) in position.

3.2 	Install the bolts (9) and the pin (10).

3.3 	With a torque wrench (Commercially available, Pt. No. std-0020),

tighten the bolts (9) from 13 to 16 lbf·in (1,47 to 1,80 Nm). Refer to

BD700-A-J20-50-00-00AAA-711A-A.

3.4 	Install the washers (8) and the screws (7).

3.5 	With a torque wrench (Commercially available, Pt. No. std-0020),

tighten the screws (7) from 13 to 16 lbf·in (1,47 to 1,80 Nm). Refer to

BD700-A-J20-50-00-00AAA-711A-A.

| 4 5 | 3.6 | If removed, install the deco panel (6). |
| --- | --- | --- |
|  | Install the applicable drawers (1). Close the nesting door. |  |

See applicability on the first page of the DM

BD700-A-J25-31-41-01AAC-720A-A

BD700-A-J25-31-41-01AAC-720A-A 
RESTRICTED 2025-05-26  Page 3

RESTRICTED

ICN-BD700-A-J253141-H-3AB48-51699-A-001-01

Figure 1 RHS 68 galley-countertop - (Sheet 1 of 2)

See applicability on the first page of the DM

BD700-A-J25-31-41-01AAC-720A-A

BD700-A-J25-31-41-01AAC-720A-A 
	RESTRICTED 	2025-05-26  Page 4

RESTRICTED

ICN-BD700-A-J253141-H-3AB48-51695-A-001-01

Figure 1 RHS 68 galley-countertop - (Sheet 2 of 2)

See applicability on the first page of the DM

BD700-A-J25-31-41-01AAC-720A-A

BD700-A-J25-31-41-01AAC-720A-A 
RESTRICTED 2025-05-26  Page 5

RESTRICTED

Requirements after job completion

Required conditions

Table 6  Required conditions

| Action/Condition | Data Module/Technical publication |
| --- | --- |

Remove all the tools, equipment, and 
unwanted materials from the work area.

Do the procedure to install the galley 
sink (25-33-21-01).

Do the procedure to install the kettle (25-
33-09-01).

Do the procedure to install the 
temperature sensor (21-61-01-06).

Do the procedure to install the wine 
drawer (25-31-27-01).

Do the procedure to install the warming 
drawer (25-33-17-01).

Do the procedure to install the entrance-area emergency-lighting exit-sign.

BD700-A-J33-51-03-01AAA-720A-A

See applicability on the 
first page of the DM 
BD700-A-J25-31-41-01AAC-720A-A

End of data module 
	RESTRICTED 	2025-05-26  Page 6